## Procedural Documentation Rubric

Learner will create a final documentation portfolio that meets the following criteria

| Criteria | Result (Achieve/ Not Achieve) |
| --- | --- |
| Lab has a title that describes its function, and the title should be a MS Word Heading | # Procedural Documentation Example 1 |
| Lab has a purpose statement that briefly explains the technologies in the lab | # Configuring Management Settings on Palo Alto |
| Lab has a logical drawing showing the topology and interfaces used in the lab. For early labs, the student copies the logical drawing from the lab sheet into the document. This often given by the instructor in this class |  |
| Lab contains either a table or embeds the information in the logical drawing that shows all the IP addresses assigned and to which devices and interfaces the addresses are assigned | Device Name Interface; Address. Inside Client; 192.168.1.20 /24. Palo Alto Ethernet 1/2 inside; 192.168.1.1 /24. Palo Alto Management Virtual Interface; 192.168.1.254 /24. Palo Alto Ethernet 1/1 outside; 203.0.113.20 /24. Palo Alto Ethernet 1/3 DMZ; 192.168.50.1 /24. DMZ Server; 192.168.50.10 /24. |
| Lab contains a step-by-step list of the commands or actions performed that are new for the semester in the current lab and on what devices they are performed. This section will use numbered bullets in MS Word. These steps should describe the tasks to complete, and the lab and should be clear enough to be used by any student who possesses the skills obtained in the Perimeter Security class. | Procedures: - To manage the Palo Alto Firewall, from the Inside Client, connect to the Palo Alto Management Interface by entering the configured Management address of 192.168.1.254 into a browser ## Configure General Firewall, and DNS and NTP Server Settings - Access the DNS server form from the DevicesSetupServices menu option and then choose the config icon in the upper right corner of the form to change the settings: - Use the form below to change your preferred and secondary DNS servers: - To configure the NTP server, click on the NTP tab and change the settings using the form below: - To change general firewall settings like domain, time zone, etc., go to DeviceSetupManagement and click on the configuration icon to access the config form: - When you are done, click commit to commit your changes and test ## Configure Management Address and Access - From the DeviceSetupInterfacesManagement Screen, click on Management: - Use the following form to set up Management Interface settings, including the address and mask, default gateway, allowed management services, network services, and permitted management workstations: - Commit your Management Interface and check ## Check for Firewall Software Updates - To check for security and software updates, choose Check Now from DeviceSoftware |
| In the step-by-step procedures, procedures are explained the first time they are used, answering the questions what the procedure is, where it is used, why it is used, and how it is used **See Appendix A Below | - To manage the Palo Alto Firewall, from the Inside Client, connect to the Palo Alto Management Interface by entering the configured Management address of 192.168.1.254 into a browser |
| Lab contains a Troubleshooting/Verification section that lists all commands/procedures, devices that the commands/procedures are on, and expected results, needed to prove the configuration works | - To verify that the DNS settings are working, you can issue a ping command, using a FQDM from the Firewall console (e.g., ping source 203.0.113.20 count 3 host www.yahoo.com) –Note that the loaded lab changes the admin password, and you will not be able to log into the firewall after you commit the load Named Config - To verify NTP, you can check the verify the time from DeviceManagementGeneral Settings screen: To verify Management address, recheck the form. Also, since Ping was allowed to the Interface, ping the address from a client to ensure that it is set up. –Note Due to the admin password change in the loaded lab, you can check to see if the browser connects to the interface, but it will not allow you to log back in. |

Appendix A

When writing procedures, we want to capture the essence of how to do the new tasks while providing enough information to configure the things we already know how to do and have documented. For example, let’s say that you are learning how to assign an interface on a firewall to a security zone. Let’s also assume that the lab tells you to set assign an IP address to the interface and you have already documented how to do that in a prior lab. In most cases, a logical drawing showing your device names and an IP address table are sufficient information to be able to set up the IP addresses and test with a ping on your PCs. You wouldn’t write step-by step procedures for this part. Instead, you want to make sure that there is enough information so that someone can place the correct address on the correct interface (answer the question, what needs to be done), and create a procedure that shows exactly how to create a security zone and add an interface to it. Your first step might say:

- Configure IP addresses on the devices as shown in the table and drawing above

What you need to document now is your new procedures (all of them). In our hypothetical lab, since creating a security zone and assigning an interface to the zone is one of our new procedures, we need to answer questions what the procedure is, where it is used, why it is used, and how it is used in our steps the first time we use the command. For example:

- To create a security zone to control traffic and for aiding in creating security policies to control the flow of traffic:

- Add your outside zone: NetworkZonesAdd  (Note that how to use the GUI was documented earlier, and one can assume that a student knows how to get to this menu. If it’s the first time all semester that you have used the GUI, how to access this should be documented as well)

- Give it the name outside and choose Layer 3 as the type. Click OK when done.

- To assign Ethernet1/1 to the outside zone and add it to the virtual router, choose: NetworkInterfacesEthernetEthernet1/1

- Configure it with the following settings:

Notice the following answers are present:

- “What procedure”: (Create Security Zone and add interface)

- “Where it’s used”: (The GUI. Note that key screenshots of forms with highlights make it easier for the user who uses your documentation)

- “Why it’s used”: (To help with security policies)

- “How it’s used”:  (The specific GUI menus and forms with their settings)

- You do this for all new procedures the first time they are used. If you need to use the procedure again in the current lab or in future labs, you only need to answer the question what needs to be done:

- Create a Security Zone Called Danger and assign Ethernet½ to this zone

Or

- Configure Zones and Interfaces as shown in the table above.