Encapsulation and Sockets Worksheet

Case 1: Frame Captured Between Web Server and Router (G0/1)

Full Socket Information:

Source IP: PC A’s IP

Source Port: Random Client Port

Destination IP: Web Server’s IP

Destination Port: 80 (HTTP) or 443 (HTTPS)

Frame Direction:

Exiting G0/1 towards the Web Server.

Justification:

Since PC A is requesting a webpage, the request must be forwarded from the client to the Web Server. This means the frame is exiting the router’s G0/1 interface heading towards the Web Server.

Case 2: Frame Captured Between Web Server and Router (G0/1)

Full Socket Information:

Source IP: Web Server’s IP

Source Port: 80 (HTTP) or 443 (HTTPS)

Destination IP: PC A’s IP

Destination Port: Random Client Port

Frame Direction:

Entering G0/1 towards PC A.

Justification:

The Web Server is sending a response to PC A’s request. Since the response is returning to the client, the frame is entering the G0/1 interface heading towards PC A.

Case 3: Frame Captured Between Web Server and Router (G0/1)

Full Socket Information:

Source IP: PC A’s IP

Source Port: Random Client Port

Destination IP: Web Server’s IP

Destination Port: 80 (HTTP) or 443 (HTTPS)

Frame Direction:

Exiting G0/1 towards the Web Server.

Justification:

The packet is moving from PC A to the Web Server, meaning it is being forwarded through the router’s G0/1 interface.

Case 4: Frame Captured Between PC A and Router (G0/0)

Full Socket Information:

Source IP: PC A’s IP

Source Port: Random Client Port

Destination IP: Web Server’s IP

Destination Port: 80 (HTTP) or 443 (HTTPS)

Frame Direction:

Entering G0/0 towards the server.

Justification:

Since PC A is making a request, the frame is moving towards the Web Server through the router’s G0/0 interface.

Case 5: Frame Captured Between PC A and Router (G0/0)

Full Socket Information:

Source IP: Web Server’s IP

Source Port: 80 (HTTP) or 443 (HTTPS)

Destination IP: PC A’s IP

Destination Port: Random Client Port

Frame Direction:

Exiting G0/0 towards PC A.

Justification:

The Web Server is sending a response back to PC A, meaning the frame is exiting the router’s G0/0 interface towards PC A.