Subnetting and Addressing Worksheet

- You are given the address range, 172.16.0.0 /16 to work with. You need four networks of equal size that can support at least 25 host addresses per network. Assuming you want to design the networks as small as possible, while still meeting your address needs, what network IDs, including subnet mask or CIDR, would you use:

  - Net 1 = 172.16.0.0 /27

  - Net 2 = 172.16.0.32 /27

  - Net 3 = 172.16.0.64 /27

  - Net 4 = 172.16.0.96 /27

- You are given the address range, 192.168.0.0 /24 to work with. You need four networks of equal size that can support at least 25 host addresses per network. Assuming you want to design the networks as small as possible, while still meeting your address needs, what network IDs, including subnet mask or CIDR, would you use:

  - Net 1 = 192.168.0.0 /27

  - Net 2 = 192.168.0.64 /27

  - Net 3 = 192.168.0.128 /27

  - Net 4 = 192.168.0.192 /27

- Given the address 172.21.32.121 /26, what is the:

  - Network ID and subnet mask of this address 172.21.32.64 and 255.255.255.192

  - What is broadcast address of this address 172.21.32.127

  - The host range of the network that this address belongs to 172.21.32.64 – 172.21.32.126

- Given the following topology, answer the questions below:

- What could be the IP address of PC A that would work? For Example 192.168.1.10

- What is the range of all addresses that would work for PC A? Usable IP range is 192.168.1.1 to 192.168.1.254

- What is the subnet mask of PC A in decimal 255.255.255.0

- What address would you configure as the default gateway for PC Typically the router’s interface on the LAN, Common choices are 192.168.1.1

- Given the following topology, answer the questions below:

- What could be the IP address of PC A that would work? For example, 192.168.2.5

- What is the range of all addresses that would work for PC A? Usable IP range 192.168.2.1 to 192.168.2.14

- What is the subnet mask of PC A in decimal 255.255.255.240

- What address would you configure as the default gateway for PC A Typically the first usable address 192.168.2.1

- Given the following topology, answer the questions below:

- What could be the IP address of PC A that would work? For example, 10.0.0.1

- What is the range of all addresses that would work for PC A? Usable Ips 10.0.0.1 and 10.0.0.2

- What is the subnet mask of PC A in decimal 255.255.255.252

- What address would you configure as the default gateway for PC A if PC A is 10.0.0.1 then the router is 10.0.0.2

- You are given the address range, 192.168.5.0 /24 to work with. You need four networks of different size that can support the following number of addresses per network:

  - Network 1 needs to support 60 host addresses

  - Network 2 also needs to support 60 host addresses

  - Network 3 needs to support 22 host addresses

  - Network 4 needs to support 11 host addresses

Assuming you want to design the networks as small as possible, while still meeting your address needs, what network IDs, including subnet mask or CIDR, would you use:

- Net 1 = 192.168.5.0/26

- Net 2 = 192.168.5.64/26

- Net 3 = 192.168.5.128/27

- Net 4 = 192.168.5.160/28