Building a Production Network Documentation:

Jobayer Bhuiyan

Purpose:

This lab demonstrates how to design and configure a production network using a Palo Alto Firewall. The lab covers:

• Configuring Source NAT for outbound Internet access.

• Configuring Destination NAT for inbound access to a DMZ web server.

• Setting up security policies that selectively allow or block applications . Allowing HTTP while blocking ping.

Logical Drawing:

Address Table:

| Device Name | Interface/Zone | IP Address | Subnet/Mask |
| --- | --- | --- | --- |
| Inside Client | Users_Net | 192.168.6.34 | 192.168.6.32/28 |
| Palo Alto – Ethernet 1/2 (Inside) | Users_Net | 192.168.6.33 | 192.168.6.32/28 |
| Palo Alto – Management | Management | 192.168.6.46 | 192.168.6.32/28 |
| Palo Alto – Ethernet 1/1 (Internet) | Internet | 203.0.113.20 | /24 |
| Palo Alto – Ethernet 1/3 (DMZ) | Extranet/DMZ | 192.168.50.1 | /24 |
| DMZ Server | DMZ | 192.168.50.10 | /24 |

Procedures:

1. Virtual Router and Routing Configuration

1. Create Virtual Router (VR-1):

• Assign the following interfaces:

• Ethernet 1/1 to the Internet zone.

• Ethernet 1/2 to the Users_Net zone.

• Ethernet 1/3 to the Extranet/DMZ zone.

2. Configure the Default Route:

• Set the destination as 0.0.0.0/0.

• Specify the next hop as the ISP/vRouter IP (e.g., 203.0.113.1).

• Choose the interface connected to the Internet (Ethernet 1/1).

2. Source NAT Configuration (Inside → Internet)

1. Create a Source NAT Rule:

• Original Packet:

• Source Zone: Users_Net

• Destination Zone: Internet

• Translated Packet:

• Translation Type: Dynamic IP and Port

• Use the IP of Ethernet 1/1 (203.0.113.20) as the translated address.

2. This rule enables the inside client to access external sites.

3. Outbound Security Policy (Inside → Internet)

1. Define a Security Policy:

• Source Zone: Users_Net

• Destination Zone: Internet

• Application: Set to “any” (or specify necessary applications)

• Action: Allow

2. This rule permits the inside client to initiate outbound connections to the Internet.

4. Destination NAT Configuration (Internet → DMZ)

1. Create a Destination NAT Rule:

• Original Packet:

• Source Zone: Internet

• Destination Zone: Internet

• Destination Address: 203.0.113.100 (public NAT IP for the DMZ web server)

• Service: tcp/80 (for HTTP)

• Translated Packet:

• Translation Type: Static IP

• Translated Address: 192.168.50.10

• Translated Port: 80

2. This rule ensures that incoming HTTP requests to 203.0.113.100 are forwarded to the DMZ server.

5. Inbound DMZ Security Policy

1. Create an Inbound Security Policy:

• Source Zone: Internet

• Destination Zone: DMZ (or Extranet)

• Initially, allow both ping (ICMP) and web-browsing (HTTP) applications.

• Action: Allow

2. This rule permits external users to test both ping and web access to the DMZ server.

6. Verification – Initial Testing

1. Outbound Test:

• From the Inside Client, access a public website (e.g., YouTube or Google) to verify outbound connectivity.

2. Inbound Tests (Before Removing Ping):

• From the vRouter (or external test device):

• Run ping 203.0.113.100 – Expect a successful ping response.

• Run curl http://203.0.113.100 – Expect to see the web page served by the DMZ server.

3. Confirm these results through CLI output or logs.

7. Remove Ping from the Security Policy

1. Edit the Inbound DMZ Security Policy:

• Remove the ping (ICMP) application from the allowed applications.

• Leave web-browsing enabled.

2. Commit the Changes:

• Apply the updated policy.

3. Final Verification:

• From the vRouter, run:

• ping 203.0.113.100 – This should now fail, confirming that ICMP is blocked.

• curl http://203.0.113.100 – This should still succeed, proving that HTTP access remains allowed.

Verification/Troubleshooting

• Monitoring:

Use the firewall’s logs (Monitor > Traffic) to verify that the NAT and security rules are functioning as expected.

• DMZ Server Check:

Ensure the DMZ server is running a web server (e.g., Apache or a Python HTTP server) on port 80 and that its default gateway is correctly set to 192.168.50.1.

• Common Issues:

If outbound or inbound traffic does not work as expected, review the NAT rules and security policies for misconfigurations or typographical errors.

Conclusion

This lab successfully demonstrates the configuration of a production network using a Palo Alto Firewall. The setup includes:

• Enabling outbound Internet access for inside clients through Source NAT.

• Providing controlled inbound access to a DMZ web server using Destination NAT.

• Implementing security policies that selectively block ping while allowing web traffic.

By following these steps and verifying each stage of the process, the network meets the lab requirements and is fully documented for procedural review.