# Basic Configuration of Palo Alto Firewall from Clean Install

Name: Jobayer Bhuiyan

## Purpose

The purpose of this lab is to perform an initial configuration of a Palo Alto firewall from a clean install. This includes resetting the device, setting up basic networking, management access, generating certificates, configuring interfaces and zones, setting up NAT, creating security policies, and verifying connectivity.

## Logical Drawing

## Address Table

| Device Name | Interface | IP Address |
| --- | --- | --- |
| Inside Client | - | 192.168.1.20/24 |
| Palo Alto | Ethernet 1/2 (inside) | 192.168.1.1/24 |
| Palo Alto | Management Interface | 192.168.1.254/24 |
| Palo Alto | Ethernet 1/1 (outside) | 203.0.113.20/24 |
| Palo Alto | Ethernet 1/3 (dmz) | 192.168.50.1/24 |

## Procedures

Launch the pod and open the firewall CLI.

Login with default credentials: admin / Pal0Alt0!

Enter maintenance mode and perform a factory reset: debug system maintenance-mode, select 'Factory Reset' > Reboot.

After reboot, login with: admin / admin, then change password to Pal0Alt0!

Generate RSA certificate: request certificate generate ca yes certificate-name PACert name paloalto.class algorithm RSA rsa-nbits 2048

Enter configuration mode: configure

Create SSL service profile and assign it: set shared ssl-tls-service-profile PA_PROFILE certificate PACert protocol-settings min-version tls1-0 max-version tls1-2

Activate SSL profile: set deviceconfig system ssl-tls-service-profile PA_PROFILE

Configure inside interface: set network interface ethernet ethernet1/2 layer3 ip 192.168.1.1/24

Assign to virtual router: set network virtual-router vm-lab interface ethernet1/2

Assign to security zone: set zone inside network layer3 ethernet1/2

Configure management IP and access: set deviceconfig system ip-address 192.168.1.254 netmask 255.255.255.0 default-gateway 192.168.1.1 permitted-ip 192.168.1.20/32

Set static management type: set deviceconfig system type static

Permit IP range: set deviceconfig system permitted-ip 192.168.1.0/24

Create interface management profile: set network profiles interface-management-profile mgmt https yes ping yes permitted-ip 192.168.1.0/24

Apply management profile: set network interface ethernet ethernet1/2 layer3 interface-management-profile mgmt

Commit the changes: commit

Access web GUI from inside client: https://192.168.1.1

Show interface info: show interface ethernet1/2

Show route info: show routing route

## Verification/Troubleshooting

- Ping test from firewall to client: ping source 192.168.1.1 host 192.168.1.20
- Ping test from client to firewall: ping 192.168.1.1
- Web GUI test: https://192.168.1.1
- Show interface config: show interface ethernet1/2
- Show routing table: show routing route
- If ping fails, check interface management profile settings and permitted IP.
- Confirm certificate and SSL profile settings.